Autonomous Intersection RSU Controller

# 1. Easy Definition of Requirements

Requirement: A clear rule that says what the system must do or how the system must behave.

Functional Requirement (FR): Describes what the system shall do. Example: The RSU shall receive vehicle requests.

Non-Functional Requirement (NFR): Describes how well, how safely, how fast, or under what constraint the system shall work. Example: The RSU shall respond within a strict deadline.

Important real-time note: Real-time behavior is mainly a non-functional timing requirement, because it defines deadlines and timing quality. However, timer handling, timeout detection, and priority handling are implemented as functional mechanisms.

# 2. System Scope

The system is an RSU-based autonomous intersection controller.

Vehicles arrive from North, South, East, and West.

Normal vehicles mainly use V2I communication: request, wait/grant, and done.

Emergency vehicles may additionally use V2V emergency beacons to warn nearby vehicles.

The RSU manages queues, checks conflicts, gives crossing permission, and prevents unsafe simultaneous movement.

M/M/1 is the baseline model with one active crossing service.

M/M/2 is the extended model with up to two safe non-conflicting simultaneous movements.

## 3. Functional Requirements (FR)

| ID | Requirement |
| --- | --- |
| FR1 | The system shall detect vehicles entering the communication zone. |
| FR2 | Each vehicle shall send a crossing request to the RSU. |
| FR3 | The vehicle request shall include vehicle ID, direction, movement type, and priority type. |
| FR4 | The RSU shall receive crossing requests from vehicles using V2I communication. |
| FR5 | The RSU shall store waiting vehicles in a queue. |
| FR6 | The RSU shall identify the vehicle direction: North, South, East, or West. |
| FR7 | The RSU shall check whether the intersection critical zone is free or occupied. |
| FR8 | The RSU shall check whether vehicle paths are conflicting. |
| FR9 | The RSU shall send GRANT only when the vehicle can safely enter. |
| FR10 | The RSU shall send WAIT when the vehicle cannot enter yet. |
| FR11 | The vehicle shall enter the intersection only after receiving GRANT. |
| FR12 | The vehicle shall send DONE after leaving the critical zone. |
| FR13 | The RSU shall remove the completed vehicle from the queue. |
| FR14 | The RSU shall select the next vehicle according to the scheduling rule. |
| FR15 | The system shall support emergency vehicle priority. |
| FR16 | Emergency vehicles shall send priority requests to the RSU. |
| FR17 | Emergency vehicles shall broadcast V2V emergency warning messages to nearby vehicles. |
| FR18 | Normal vehicles shall wait or yield when an emergency vehicle is active. |
| FR19 | After the emergency vehicle exits, the RSU shall return to normal queue operation. |
| FR20 | The system shall provide simulation output such as active grant, queue state, and waiting vehicles. |

## 4. Non-Functional Requirements (NFR)

| ID | Requirement |
| --- | --- |
| NFR1 | The system shall be collision-free inside the critical intersection zone. |
| NFR2 | Two conflicting vehicles shall never receive crossing permission at the same time. |
| NFR3 | The system shall be deadlock-free. |
| NFR4 | The system shall prevent starvation, so no vehicle waits forever. |
| NFR5 | The RSU shall make each decision within a strict real-time deadline. |
| NFR6 | The system shall process emergency vehicle requests with higher priority than normal requests. |
| NFR7 | Emergency priority shall not violate collision safety. |
| NFR8 | The system shall minimize average waiting time where possible. |
| NFR9 | The system shall reduce unnecessary congestion at the intersection. |
| NFR10 | The system shall behave deterministically, so the same input scenario gives the same decision. |
| NFR11 | The system shall be modular, separating queue logic, conflict checking, timing, and communication. |
| NFR12 | The system shall be simple enough to model in UPPAAL. |
| NFR13 | The system shall be implementable later using C-code and FreeRTOS. |
| NFR14 | The system shall be suitable for hardware-level simulation using ModelSim. |
| NFR15 | The system shall recover to a safe state after reset. |
| NFR16 | The system shall handle lost or delayed messages safely. |
| NFR17 | The system shall not grant a new vehicle if the previous vehicle has not completed crossing. |
| NFR18 | The system shall support comparison between M/M/1 and M/M/2 queueing models. |

## 5. Extra Requirements for M/M/1

| ID | Requirement |
| --- | --- |
| MM1-FR1 | The RSU shall allow only one vehicle movement at a time. |
| MM1-FR2 | All other vehicles shall wait while one vehicle is crossing. |
| MM1-FR3 | The intersection state shall be either FREE or OCCUPIED. |
| MM1-NFR1 | M/M/1 shall be used as the simple baseline model. |
| MM1-NFR2 | M/M/1 shall prioritize safety over throughput. |

## 6. Extra Requirements for M/M/2

| ID | Requirement |
| --- | --- |
| MM2-FR1 | The RSU shall support up to two simultaneous vehicle movements. |
| MM2-FR2 | The RSU shall allow two vehicles only if their paths are non-conflicting. |
| MM2-FR3 | The system shall maintain two service slots, for example server_1 and server_2. |
| MM2-FR4 | The RSU shall check the conflict matrix before granting the second vehicle. |
| MM2-NFR1 | M/M/2 shall improve throughput compared to M/M/1. |
| MM2-NFR2 | M/M/2 shall not reduce safety compared to M/M/1. |
| MM2-NFR3 | If no safe second vehicle exists, the second service slot shall remain unused. |

# 7. Short Explanation for Presentation

FR = what the RSU does. NFR = how safely, fast, reliably, and efficiently the RSU does it.

Example: "The RSU shall send GRANT or WAIT" is an FR. "The RSU shall send the response within a strict deadline" is an NFR because it is a timing constraint.

M/M/1 vs M/M/2: In M/M/1, only one vehicle movement is allowed at a time. In M/M/2, two movements may be allowed at the same time only if their paths are non-conflicting. Therefore, M/M/2 needs extra conflict-checking and service-slot requirements.